Notes:

- If the warehouse is large BLE Mesh might be better to replace BLE. (4)

- Keep adjacent wireless hops relatively short ~15 meters. (4)

- connect: network PLR → missing sensor samples → degraded forecast reliability. (4)

- Talk about the possibility of using a sniffer (nRF52840 USB dongle). (4)

- Why did I use feather Nrf52840 Express & SHT45 instead of ESP8266 & DHT11. (6, 7, 8, 9)

- Why BLE not WiFi kits. (17 -wifi consumes a lot of energy and needs all nodes to be connected via an ip), (18 - power outcomes depend on how frequently nodes reconnect, how long radios stay awake, and whether a gateway can keep links stable)

- Why (Grafana/Node-RED/Flask/Adafruit IO) for a dashboard. (12), (15), (16 -Adafruit IO)

- Discuss managing warehouse stock through IoT system that detects temp and humidity. (1)

- Why only measure temp/rh for spices. (14 -they affect water activity, mold risk, and quality degradation), (21 -they influence water activity, moisture sorption, deliquescence, and glass transition), (28)

- Discuss end-to-end pipeline: pod –> gateway –> dashboard. (5)

- Justify why prediction + better sensing is needed. (1)

- Discuss “Edge Computing = Gateway”. (3), (2)

- Example of “edge-enabled smart warehouse shelves”. (3)

- Why Use of Adafruit IO. (3), (11)

- Discuss the forecasting accuracy. (13)

- Static thresholds are not enough –> multi-point sensing. (2), (27 - RH naturally drifts with time-of-day/operations; alerting must adapt to baseline patterns to avoid noise)

- Discuss deploying multiple pods across the warehouse, near gates, vents, etc. airflow patterns + historical hotspots + storage sensitivity. (2)

- Explain data quality pipeline for ML: cleaning + robustness. (2)

- Justify threshold-based labelling as a valid starting point for supervised learning. (2)

- Dynamic warehouse environments; need for more than manual inspection/threshold rules. (2)

- Discuss CBR + fuzzy reasoning. (5)

- Mirror evaluation metrics. (5)

- Use a data pipeline of: Sensors -> preprocessing -> ML (forcasting/anomaly detection) -> -> Logging (csv) -> Dashboard / Alerts. (12), (26 -logging is also for detecting drifts not just for plotting)

- Simple, mobile-friendly dashboards for human-in-the-loop interfaces. (10)

- Data logging systems. (10)

- TEC + relay consumes a lot of energy, thus pods do not drive heavy actuators. (10)

- Use of Database. (1)

- Use of MAE, and RMSE. (13)

- Use of lightweight ML (VAR) for less computational power. (13)

- Train multiple times and report average error. (13)

- Compare “Last value persists”, VAR/ARIMA, and LSTM/GBDT. (13)

- Damaged stock result in loss of aroma/volatiles, not just mold. (14)

- RH ~65%, > 22 C temp thresholds. (14)

- We can apply our IoT solution instead of changing the storage method (hermetic, vacuum, CO2) to preserve quality. (14)

- Why need a dashboard. (15)

- Time-series, streaming, and prescriptive analytics. (15)

- Control transmission frequency and payload size. (17)

- BLE pods + gateway is more scalable and energy-aware design. (17)

- Managing wi-fi for many distributed pods adds: Wi-Fi credential provisioning, network coverage/dead zones, AP congestion with many clients, security management, and power tradeoffs (stay connected vs reconnect). (18)

- Sampling/publishing strategy. (18)

- BLE vs WI-FI power outcomes depend on duty cycle and reconnection strategy. (18)

- Data quality, calibration checks, filtering, and smoothing. (19)

- Food safety and quality context for spices: Cite their findings that category/source/FSMS affect microbiological quality and that bulk/unpackaged tends to be worse. (20)

- Justification of Monitoring + logging: Connect FSMS principles: controlled storage conditions, record keeping, and early corrective action. (20)

- Why forecasting matters, as short-term excursions in RH/temp can push a product across Tg/RH0/crossover boundaries, causing rapid physical change. (21)

- Metal and moving stock can significantly affect BLE signal reception and system performance. (22)

- BLE reliability affects forecast quality, coexistence risks data quality, should log “missing samples”. (23)

- Future work: measuring interference, and move toward adaptive channel selection / coexistence-aware configuration. (23)

- Scalability of BLE central/gateway challenges: resource overlap and unfair scheduling can make the system unstable even if connections technically exist. (24)

- Discuss clock drifts, and the need to correct timestamps across multiple nodes through a lightweight application-layer time sync. (25)

- Static offline models degrade under drifts. (26)

- Include threshold-based labeling as a baseline, to help with anomalies and concept drift. (26)

- Alert fatigue -> temporal aggregation. (27)

- Most IoT food monitoring focuses on storage/transport. (28)

- 4 layer IoT architecture: sensing layer (pods) -> communication layer (BLE to Gateway + ML) -> storage layer (CSV) -> application/control layer (Dashboard). (28)

- Environmental monitoring is an explicit scheme requirement (for specific categories -not RH and temp). (29)

- We can calculate dew point/vapor-pressure features through T and RH only using MBE/RMSE to evaluate estimation accuracy. (30 -extra parameters without adding extra sensors)

Sentences for each source:

(1):

- An IoT-assisted warehouse monitoring approach can support stock preservation by continuously sensing temperature and humidity (moisture)—identified as crucial quality factors—and communicating abnormal conditions to responsible staff so timely corrective action can be taken to prevent degradation and waste.

- Because the authors explicitly note opportunities to improve warehouse outcomes via higher-sensitivity, more reliable sensors and further enhancement using machine-learning techniques (e.g., neural networks), this motivates adding prediction and improved sensing to move from reactive alarms to earlier, risk-reducing intervention.

- A database is justified as the backbone for remote monitoring and traceability, since the system architecture stores and streams sensed readings to a real-time database (Firebase) (and web platforms) to enable persistent records, visualization, and decision support.

(2):

- Their deployment strategy places multiple sensor pods in key zones selected by airflow patterns, historical fluctuation “hotspots,” and storage sensitivity (e.g., near vents, exits/gates, and dense shelving) to capture localized changes.

- To make ML reliable, they emphasize a data-quality pipeline where raw multi-sensor streams are strengthened using filtering/smoothing, outlier detection and repair, and dimensionality reduction (e.g., PCA) before modelling.

- They show that threshold-based labelling is a valid supervised-learning starting point by annotating historical records using predefined “acceptable ranges” (e.g., temperature and humidity bands) to create an initial ground truth for training and evaluation.

- Because automated warehouses are dynamic and complex, the authors conclude that manual inspection and fixed rule/threshold control cannot adapt well, motivating predictive and adaptive methods (e.g., deep learning with fuzzy control) driven by real-time and historical data.

(3):

- They illustrate an edge-enabled smart warehouse shelf concept where an ESP32-based node monitors environmental factors (e.g., temperature and humidity) and supports food-safety actions via local sensing plus connected notifications and dashboards.

(4):

- For large warehouses where single-link BLE coverage becomes unreliable, the source shows that BLE Mesh adds a many-to-many, multi-hop communication model that scales to large node counts and extends monitoring range through relays.

- The experimental results indicate that keeping adjacent hop distances around ~15 m maintains strong RSSI and low packet loss, while longer indoor links (≈30 m+) rapidly degrade into very high PLR or no signal.

- The thesis operationalises Packet Loss Ratio (PLR) as “missed packets,” so higher PLR directly implies missing sensor samples, which would reduce the integrity of any downstream forecasting model that depends on continuous time-series data.

- A practical way to debug link quality is to use an nRF52840 USB dongle as a sniffer with Wireshark to capture BLE Mesh traffic and inspect fields such as TTL/SRC to verify relaying and diagnose loss.

(5):

- The paper’s RIQS architecture demonstrates an end-to-end operational flow—data capture at the warehouse interface, storage/decoding in a central database, then decision outputs displayed via a web interface—which aligns with a pod → gateway → dashboard pipeline in our system.

- It justifies combining CBR + fuzzy reasoning by using CBR to retrieve the most similar historical handling cases and fuzzy reasoning to translate imprecise, expert-style quality concepts into actionable storage and handling recommendations.

- Their evaluation quantifies “before vs after” impact using operational KPIs (e.g., order-fulfilment accuracy, decision-time reduction, customer complaints, and defective inventory rate), providing a template for mirroring comparable metrics when assessing our predictive monitoring workflow.

(6, 7, 8, 9):

- I chose the Feather nRF52840 Express + SHT45 because it provides a BLE-capable MCU platform (with native USB/UF2 and ample memory) and a high-accuracy, I2C temperature/RH sensor (±0.1 °C typical, ±1.0%RH typical), whereas ESP8266 + DHT11 would rely on Wi-Fi connectivity and a lower-grade sensor with narrower operating range and coarser accuracy (e.g., 0–50 °C and ±1 °C).

(10):

- The authors demonstrate a simple, mobile-friendly human-in-the-loop dashboard by using the Blynk iOS/Android app (drag-and-drop widgets) to show live temperature/RH graphs and gauges and to let a user manually issue control commands.

- They recommend adding a data-logging system so that when internet connectivity is unavailable, sensor readings can still be continuously recorded for later analysis.

- Because their prototype shows that a TEC is the highest power load (12V, 15A) and can stress/fail the relay when switched, this supports designing our pods as sensing/communication devices rather than driving heavy actuators directly.

(11):

(12):

(13):

- The source assesses forecasting accuracy by directly comparing predicted versus actual environmental sensor readings (temperature, humidity, CO₂) and reporting the resulting prediction errors.

- It uses MAE and MSE (and by extension RMSE as the square root of MSE) as standard regression/time-series error metrics to quantify model performance.

- The experiments show that a lightweight VAR model can train orders-of-magnitude faster than LSTM while still achieving competitive accuracy in some cases, supporting VAR as a practical option for computationally constrained deployments.

- To improve result reliability, each model is trained 30 times with fixed parameters and the average error (and processing time) is reported rather than relying on a single run.

- The study demonstrates the importance of benchmarking simple statistical models (VAR, and it also discusses ARIMA as a common alternative) against deep models (LSTM/CNN), motivating a structured comparison of baselines versus complex learners when selecting a forecasting approach.

(14):

- The source shows that storage quality loss is not limited to mold or pests, because aroma and taste can degrade when volatile compounds escape during aerated storage, reducing organoleptic quality even if the product is otherwise “dry.”

- It reports that mold activity begins when intergranular humidity rises above ~65% RH and explains that higher temperature increases vapor pressure and accelerates volatilisation, supporting the use of humidity/temperature limits in preservation monitoring.

- Because the paper finds hermetic, vacuum, and high-CO₂ storage preserve aroma far better than aerated storage, a practical implication is that an IoT monitoring system can help maintain comparable quality by detecting harmful humidity/temperature drift early and enabling timely corrective actions without necessarily changing the storage method.

(15):

- A dashboard is needed because IoT deployments generate large, varying data that must be made understandable and actionable, enabling operators to monitor conditions centrally and rely on alerts instead of manual, continuous checking.

- The reference distinguishes time-series analytics (pattern extraction over time), streaming analytics (analysis of in-motion data to detect critical situations), and prescriptive analytics (recommending the best action), which maps directly to sensor history visualisation, real-time threshold alerts, and recommendation outputs in our system.

(16):

(17):

- It emphasises that wireless radio activity dominates energy consumption, therefore battery life can be improved by controlling transmission frequency and the amount of data sent (payload/packet size and number of packets) while still meeting application QoS.

- Since Wi-Fi connection/association has a high energy cost and Wi-Fi is power-hungry compared to low-power IoT radios, using BLE pods that relay to a mains-powered gateway (which handles Internet backhaul) is a more scalable and energy-aware architecture for warehouse sensing.

(18):

- At scale, Wi-Fi sensing implies managing persistent IEEE 802.11 connectivity (TCP/IP association), and the paper highlights that Wi-Fi’s practical viability for IoT hinges on careful power design rather than assuming “always-on” connections are free.

- Their experimental setup demonstrates a sampling/publishing strategy where transmissions are triggered only when required (instead of continuously), because constant sending is impractical for battery devices and increases power draw.

- Overall, the results reinforce that BLE vs Wi-Fi power outcomes depend on duty cycle and reconnection strategy, since maintaining a link versus repeatedly establishing/de-establishing it drives the per-report energy cost across different reporting intervals.

(19):

- The source demonstrates that maintaining sensor data quality requires calibration checks against a higher-precision reference and applying software compensation (offset correction via curve fitting) to reduce measurement error, alongside routine filtering/smoothing to stabilise temperature–humidity readings for reliable monitoring and control.

(20):

- The study on spices and dried herbs found that microbiological quality varies significantly by source/packaging and the presence of a Food Safety Management System (FSMS), with imported and FSMS-certified packaged products showing lower microbial loads, while unpackaged/bulk products had the highest contamination, highlighting a clear food-safety and quality risk for bulk spices.

- This evidence supports monitoring and logging because the authors emphasise FSMS/HACCP-aligned controls—controlled storage conditions, good hygienic/manufacturing practices, process controls, and routine inspection/testing—to detect issues early and trigger corrective action, which directly maps to continuous sensing plus recorded environmental data for traceability.

(21):

- Forecasting matters because even short RH/temperature excursions can cross Tg or the aw–RH₀ crossover point, triggering rapid physical destabilisation (plasticisation, deliquescence, liquid bridges) and therefore sudden caking or texture change.

(22):

- In real warehouses, metal infrastructure and moving metallic stock create highly dynamic multipath channels that block/reflect/refract BLE signals, which the authors show can significantly degrade performance (e.g., noticeably higher positioning error when a pallet carries steel sheets versus an empty pallet).

(23):

- Because Wi-Fi and BLE share the 2.4 GHz ISM band, coexistence interference can corrupt packets and trigger failures/retries, which effectively creates missing sensor samples and should therefore be logged explicitly (e.g., “missing samples” / PLR) since those gaps propagate into weaker time-series forecasting reliability.

- The paper motivates future work toward interference measurement and coexistence-aware adaptation by building an interference map from spectrum observations and then applying adaptive channel selection / frequency-hopping strategies to minimise mutual Wi-Fi–BLE interference while maintaining high connection success.

(24):

- The paper shows that scaling a BLE central to many simultaneous peripherals is challenging because connection-event resource overlap and the resulting unfair scheduling (blocking/preemption) can cause QoS degradation, instability, and even disconnections, meaning a system may appear “connected” yet still behave unreliably under load.

(25):

- Because independent sensor nodes experience clock drift that can accumulate into millisecond-level timestamp misalignment over typical sensing durations, the source motivates a lightweight gateway-driven synchronisation approach (e.g., periodic beacons to estimate drift rate and then compensate data timestamps across nodes at the application level) to keep multi-node measurements time-aligned.

(26):

- The study shows that static batch (offline) models lose robustness when concept drift occurs, motivating either frequent retraining or streaming/online updates for long-running deployments.

- Since the paper frames anomaly detection as binary classification in data streams under concept drift, a pragmatic baseline is to label threshold exceedances as “anomalous” to bootstrap supervised learning and to provide an interpretable reference for tracking drift-related performance drops.

(27):

- To mitigate alert fatigue, the study supports temporal alert aggregation—grouping related alerts within a time window—so operators receive fewer, more contextual incident notifications instead of many repetitive threshold-triggered alarms.

(28):

- The review shows that most IoT food-monitoring work targets storage and transportation (storage 38% and transportation 37% of studies), reflecting that monitoring efforts concentrate primarily on these stages of the supply chain.

- Using the standard 4-layer IoT architecture, our system maps pods to the sensing layer, BLE-to-gateway (edge) to the communication layer (via gateways), CSV logging to the storage layer, and dashboard/alerts to the application-and-control layer for decision support and performance visualisation.

(29):

- FSSC 22000 (v6) makes environmental monitoring an explicit scheme requirement for specific food chain categories (BIII, C, I and K) by mandating a risk-based program targeting relevant pathogens/spoilage/indicator organisms, with documented control-effectiveness evaluation, trended monitoring data, and periodic review.

(30):

- Psychrometric features such as dew-point temperature (and water-vapour pressure via RH as the ratio of partial vapour pressure to saturation pressure) can be derived from temperature and relative humidity alone without additional sensors, and the estimation model can be validated using MBE and RMSE as demonstrated in the paper’s dew-point model comparison against psychrometric-table ground truth.

- For spices, monitoring only temperature and RH is well-motivated because they are the most common food-chain variables and they directly govern moisture sorption/water activity and stability boundaries (e.g., deliquescence RH₀ and glass transition Tg) that drive rapid quality loss (21, 28).

- Edge computing effectively plays the role of the warehouse gateway by placing computation/storage near the sensors to perform low-latency local preprocessing and analytics before optionally forwarding to the cloud (2, 3).

- Static thresholds and single-point sensing are inadequate in warehouses due to spatial variability and baseline drift, so the literature supports distributed multi-point sensing with adaptive thresholding to reduce missed events and false-alarm noise (2, 27).

- An end-to-end pipeline of Sensors → preprocessing → ML (forecast/anomaly) → logging (CSV/DB) → dashboard/alerts matches established IoT/streaming architectures, with logging serving both historical analysis and detection of drift/missingness that degrades model reliability (12, 26).

- BLE is preferable to Wi-Fi for battery pods because Wi-Fi is comparatively power-hungry and requires router/IP association, while overall radio energy depends heavily on duty cycle and connection/reconnection strategy (17, 18).

Several dashboard options (e.g., Grafana/Node-RED/Flask/Adafruit IO) can implement the IoT monitoring/visualisation layer for time-series/streaming data with real-time status, KPIs, and predictive alerts via dashboards/APIs, but in this project I choose Adafruit IO because it offers a managed MQTT/HTTP feed backend with built-in dashboards for rapid logging/visualisation and has been validated in an edge-enabled warehouse monitoring prototype with alerting integration (12, 15, 16, 3, 11).

Sources:

- https://link.springer.com/chapter/10.1007/978-981-19-5845-8_16

- IoT-Based Multi-Sensor Environmental Monitoring and Intelligent Control in Automated Warehouses Using Fuzzy Logic and Deep Learning | Zhang | Informatica

- Edge Computing Enabled Smart Warehouse Management System for Food Processing Industries | IEEE Conference Publication | IEEE Xplore

- Development and optimization of a Bluetooth Low Energy (BLE) Mesh network for environmental monitoring and alerting - Theseus

- Real‐time inbound decision support system for enhancing the performance of a food warehouse | Journal of Manufacturing Technology Management | Emerald Publishing

- https://www.adafruit.com/product/4062?srsltid=AfmBOorfKQullS1Da2Wz8Efc_5pKPY9eXl7-T2d6orzONiCLW2AWKyIb

- https://www.adafruit.com/product/5665?srsltid=AfmBOooRYpj_UGYYXVxmq71WkWALRBlR2Z3bpDo_oYSYkliwXdHucTBd

- https://www.espressif.com/en/products/socs/esp8266

- https://components101.com/sensors/dht11-temperature-sensor

- https://ieeexplore.ieee.org/document/10033410

- https://io.adafruit.com/

- https://www.mdpi.com/1424-8220/25/9/2945

- https://ieeexplore.ieee.org/abstract/document/10135031?casa_token=rCbDOWFosa8AAAAA:TcQ2L4OldOnlbzpfjdfivtd-S0PK9PMTHtiImcVqBo6bpMRQheSHlmxW9dtqFkQXIB8RaOX3Uw4

- https://d1wqtxts1xzle7.cloudfront.net/82973895/9803-libre.pdf?1648717326=&response-content-disposition=inline%3B+filename%3DThe_significance_of_vapor_pressure_in_qu.pdf&Expires=1768865943&Signature=Mx2Qrx7CutlZ7uyb9pwUYkTGtkELgiAvkDyMa0rWAh38kjzVB6l3LsHBpF9t4RlnnYDBG5ZcUaAixTW79MMzKE6aMZNX94OxCac1PlafWvGpo3TqBMD9ujVOhs2Ef7Gebc6LgqFVgvQUe64mAPOE~N7u02C-VMMBuvtNFSUoZwFaWarj87pUKmLJiOXU~Gx1VIf5U6PfXtEZRGYm8k7Ua6xAWFsL3ZTvTE01dRuRMWhvuUgrrkjzYa3pvxvkfGivDqBMboMhwHNdfsJpMZqLj6LG-oZYILYUmd07eeffbgHNWne7xHesoUMHfJYC1K4~bIKdIGlJawewkxAddWHqrA__&Key-Pair-Id=APKAJLOHF5GGSLRBV4ZA

- https://link.springer.com/chapter/10.1007/978-981-15-2282-6_7

- https://ieeexplore.ieee.org/stamp/stamp.jsp?tp=&arnumber=9872787

- https://electricajournal.org/public/pdfs/86/62-70(2).pdf

- https://d1wqtxts1xzle7.cloudfront.net/100577118/10005628.pdf?1680441808=&response-content-disposition=inline%3B+filename%3DComparison_Of_Power_Consumption_Of_Wifi.pdf&Expires=1768869393&Signature=CaJduwqmIq13AdN6cIHOzR12gbBtZwdStgk-uPPybP~SuejeaFrjgNVsS3yAN9t0Rdc2EXH3Cmpo9TnIxXnnBacfhSViMVvayBw6CroUDDE4VOyNKBkk-ZswWbYpSmj59Z2U6d3lua79DC9PFDVBjD7ic0H-ZQjWU0-F272D0nl4lT-MgvpP7p10X4qWMS8p2Bm38cB59y8wGNXBJqap5-0stvLavcJdF2h7Ciw7Wz2mZG3KKwfdPWCA100VBiDO-3dfDeSppan65wAzEI0K6ZcFq3ArrK1V34s48Isvgnyza44FJV6P0jOdunldi3XXm1TJmyt9sYSLDg8PjATaQw__&Key-Pair-Id=APKAJLOHF5GGSLRBV4ZA

- https://ieeexplore.ieee.org/stamp/stamp.jsp?tp=&arnumber=9426002

- https://www.sciencedirect.com/science/article/pii/S0168160521002543?casa_token=YIj6QHtlp2UAAAAA:IjwXIzfdpd-oJAUCm9V6hNSOEqYgmQ_uWJqIrm7MK3g4hSnO3lzK1KApR4cvMif7wQd0rCC5yOM

- https://www.sciencedirect.com/science/article/pii/S0963996920306335?casa_token=PNdAsKtCUYwAAAAA:hP6BWZbxRtta74ZpZTwgzs_Ae0WIzz0cB-Igp_0EAcB7xOpkVAmRH7W75ef1aXwnoHbjylIJT-M

- https://ieeexplore.ieee.org/stamp/stamp.jsp?tp=&arnumber=10817464

- https://www.mdpi.com/1424-8220/23/11/5183

- https://ieeexplore.ieee.org/abstract/document/11275955

- https://onlinelibrary.wiley.com/doi/full/10.1002/stc.1782

- https://arxiv.org/abs/2510.27304

- https://ciajournal.com/index.php/jcia/article/view/28

- https://www.mdpi.com/2071-1050/15/1/614

- https://www.fssc.com/wp-content/uploads/2023/03/FSSC-22000-Scheme-Version-6-Final.pdf

- https://czasopisma.uwm.edu.pl/index.php/ts/article/view/5425