Project Proposal: Predictive Food-Stock Preservation System for Warehouse Environments

1. Overview

This project aims to design and prototype an IoT-based intelligent monitoring system that helps prevent spoilage and quality degradation of food stock in warehouse environments—particularly products that are sensitive to humidity and temperature, such as dry spice kits and semi-wet paste kits. It integrates low-cost sensor nodes, a gateway, machine-learning-based risk prediction, and a web dashboard for quality-control (QC) supervisors.

2. Objectives

- Continuous Monitoring: Capture real-time temperature and relative humidity from multiple micro-zones (“pods”) within a warehouse.

- Prediction: Use machine learning to forecast the likelihood of environmental conditions exceeding spoilage-risk thresholds (e.g., RH > 65 %, temperature > 22 °C) within the next 30 minutes.

- Alert & Recommendation: Provide actionable insights—such as when to dehumidify, adjust ventilation, or relocate stock.

- Data Visualization: Offer supervisors a live dashboard displaying readings, historical trends, predicted risks, and recommended actions.

3. Hardware Architecture

| Component | Model / Example | Function |
| --- | --- | --- |
| Sensor Node (Pod) | Adafruit Feather Express nRF52840 Board + SHT45 Temp/RH sensor | Reads temperature and relative humidity every 1 minute. Sends data to the gateway via Bluetooth Low Energy (BLE). |
| Gateway | Developer’s laptop | Acts as a BLE central device. Collects sensor data from the pod, runs ML models and decision logic, then forwards summarized results to the dashboard through Wi-Fi. |

4. System Components & Their Roles

- Sensor Node (Pod):

  - Measures and timestamps temperature (°C) and relative humidity (%) in its local micro-zone.

  - Computes quick statistics (e.g., minutes > threshold) and transmits them via BLE.

  - Designed for low power and low cost (~£60 per node).

- Gateway:

  - Receives data from the pod through BLE using Python’s bleak library.

  - Buffers readings, performs basic cleaning, and executes the ML model to estimate risk of spoilage for the next 30 minutes.

  - Publishes readings and predictions via MQTT/HTTP over Wi-Fi to the web dashboard.

- Machine Learning Model:

  - Trained on public indoor-environment datasets.

  - Learns humidity-temperature dynamics and predicts when risk thresholds will likely be exceeded.

  - Initially uses rule-based threshold labels; future versions will refine predictions using real stock-damage records.

- Web Dashboard:

  - Web-based interface served from the gateway or a local server.

  - Built with Grafana, flask, or optionally Adafruit IO for interactive visualization.

  - Displays: current readings, historical graphs, predictive risk (green/amber/red), and textual recommendations for QC staff.

  - Logs all data for long-term analysis and system improvement.

5. Expected Outcomes

- A working hardware–software prototype demonstrating end-to-end data flow: Pod → Gateway → Dashboard.

- A trained ML model capable of short-term (≈ 30 min) prediction of humidity/temperature risk levels.

- A web dashboard presenting clear, actionable insights for warehouse quality-control operations.